Development Log for Software Design

Project Title: Power Quality Waveform Analyser
Date Started: January 29, 2026
Development Period: 14 Weeks.
Author: Nlemchi Ebubechukwu David

Student ID: 25059498

- Structure Chart
A structure chart is created to visually represent the different modules in the system. Below
is a simple structure chart of the system:

- Program Design Pseudocode

START

Allocate memory for 1000 Dataset records

IF memory allocation fails THEN

Display memory allocation error

End program

END IF

Open power_quality_log.csv

IF file cannot be opened THEN

Display file opening error

Free allocated memory

End program

END IF

Skip the CSV header row

Set sample counter to 0

WHILE sample counter is less than MAX_SAMPLES AND row has 8 valid values

Read timestamp

Read Phase A voltage

Read Phase B voltage

Read Phase C voltage

Read current

Read frequency

Read power factor

Read THD

Store values in Dataset array

Increase sample counter

END WHILE

Close CSV file

IF no samples were read THEN

Display no samples error

Free allocated memory

End program

END IF

Open results.txt for writing

FOR each phase: Phase A, Phase B, Phase C

Calculate RMS voltage

Calculate DC offset

Find maximum voltage

Find minimum voltage

Calculate peak-to-peak voltage

Count clipping events

Write phase results to results.txt

END FOR

Calculate average frequency

Calculate average power factor

Calculate average THD

Write overall measurements to results.txt

Close results.txt

Free allocated memory

Display completion message

END

- Function Description

- Function: main(), version 1.0

Description:

-Controls the overall program flow. It allocates memory, loads the CSV file, checks that data was read, calls the report function, frees memory, and ends the program.

Input Parameters:

-None.

Returns:

-int

-Returns 0: if the program runs successfully.

-Returns 1: if memory allocation fails or no samples are read.

- Function: LoadCSV(), version 1.0

Description:

-Opens the CSV file, skips the header row, reads each row of waveform data, and stores the data inside an array of Dataset structures.

Input Parameters:

-const char *filename

-The name of the CSV file to open.

-Dataset *data

Pointer to the array where the CSV data will be stored.

Returns:

-int

-Returns the number of valid samples successfully read from the file.

- Function: WriteReport(), version 1.0

Description:

-Processes the loaded dataset and writes the final analysis results to results.txt.

Input Parameters:

-Dataset *data

Pointer to the array of loaded samples.

int n

-Number of samples read from the CSV file.

Returns:

-void

This function does not return a value. It writes the results directly to a text file.

- Function: WritePhaseReport(), version 1.0

Description:

-Writes the waveform analysis results for one phase into results.txt.

Input Parameters:

-FILE *file

Pointer to the output file.

-const char *phaseName

Name of the phase being analysed, for example Phase A.

-double values[]

Array of voltage samples for one phase.

int n

-Number of samples in the array.

Returns:

-void

This function does not return a value. It writes results to the output file.

- Function: calculateRMS(), version 1.0

Description:

-Calculates the RMS voltage of a waveform.

Input Parameters:

-double values[]

Array of voltage samples.

int n

-Number of samples in the array.

Returns:

-double

Returns the RMS voltage value.

- Function: calculateMean(), version 1.0

Description:

-Calculates the mean voltage of the waveform. This is used to detect DC offset.

Input Parameters:

-double values[]

Array of voltage samples.

-int n

Number of samples in the array.

Returns:

-double

Returns the average voltage value.

- Function: peak_peak(), version 1.0

Description:

-Finds the maximum and minimum voltage values in a waveform.

Input Parameters:

-double values[]

Array of voltage samples.

int n

-Number of samples in the array.

-double *max

Pointer used to store the maximum value.

-double *min

Pointer used to store the minimum value.

Returns:

-void

This function does not return a value directly. The maximum and minimum values are returned through pointers.

- Function: count_clipping(), version 1.0

Description:

-Counts the number of waveform samples that reach or exceed the clipping threshold.

Input Parameters:

-double values[]

Array of voltage samples.

int n

-Number of samples in the array.

Returns:

-int

Returns the number of clipped samples detected.

- Evidence of Unit Testing

- Unit Testing Function: calculateRMS(), version 1.0

Test Case 1:

Input:

values[] = {3.0, 4.0}

n = 2

Expected Output:

3.54

Reason:

RMS = sqrt((3² + 4²) / 2)

RMS = sqrt((9 + 16) / 2)

RMS = sqrt(12.5)

RMS = 3.54

Result:

Passed

Test Case 2:

Input:

values[] = {230.0, 230.0, 230.0}

n = 3

Expected Output:

230.00

Result:

Passed

- Unit Testing Function: calculateMean(), version 1.0

Test Case 1:

Input:

values[] = {10.0, -10.0, 20.0, -20.0}

n = 4

Expected Output:

0.00

Result:

Passed

Test Case 2:

Input:

values[] = {5.0, 10.0, 15.0}

n = 3

Expected Output:

10.00

Result:

Passed

- Unit Testing Function: peak_peak(), version 1.0

Test Case 1:

Input:

values[] = {-325.0, -100.0, 0.0, 100.0, 325.0}

n = 5

Expected Output:

Maximum = 325.00

Minimum = -325.00

Result:

Passed

Test Case 2:

Input:

values[] = {4.0, 9.0, 2.0, 15.0, -3.0}

n = 5

Expected Output:

Maximum = 15.00

Minimum = -3.00

Result:

Passed

- Unit Testing Function: count_clipping(), version 1.0

Test Case 1:

Input:

values[] = {100.0, 324.9, -324.9, 200.0}

n = 4

Expected Output:

2

Reason:

324.9 and -324.9 both meet the clipping condition. Which is to flag any sample where |voltage| >= 324.9v

Result:

Passed

Test Case 2:

Input:

values[] = {100.0, 200.0, -300.0}

n = 3

Expected Output:

0

Result:

Passed

- Unit Testing Function: LoadCSV(), version 1.0

Test Case 1:

Input:

Valid CSV file containing 1000 data rows.

Expected Output:

1000 samples read successfully.

Result:

Passed

Test Case 2:

Input:

Missing CSV file.

Expected Output:

Program displays file opening error.

Result:

Passed

Unit Test Results Summary:

-The main calculation functions were tested using small datasets where the expected results could be calculated manually.

-The RMS calculation produced correct values for both simple values and repeated voltage values.

-The mean calculation was tested using balanced positive and negative values to confirm that DC offset was correctly calculated.

-The peak_peak() function correctly identified maximum and minimum values.

-The count_clipping() function correctly detected samples greater than or equal to ±324.9 V.

-The CSV loading function was tested using the real dataset and also tested with a missing file to confirm error handling.

Remaining Issues:

-No major issues remain in the current version.

Future Improvements:

-Further tests could be added for corrupted CSV rows, empty files, and datasets larger than MAX_SAMPLES.

- Evidence of Integration Testing

Integration Test 1:

Functions tested together:
-main() version 1.0 and LoadCSV() version 1.0

Test Steps:
1. Run the program from main.c.
2. Allocate memory for the Dataset array.
3. Call LoadCSV() using power_quality_log.csv.
4. Check the number of samples read.

Expected Outcome:
-The program should load the CSV file successfully.
-LoadCSV() should return 1000 samples.

Result:
Passed


Integration Test 2:

Functions tested together:
-LoadCSV() version 1.0 and WriteReport() version 1.0

Test Steps:
1. Load the CSV file using LoadCSV().
2. Pass the loaded Dataset array into WriteReport().
3. Generate results.txt.
4. Open results.txt and check the output.

Expected Outcome:
-The program should create results.txt.
-The report should contain results for Phase A, Phase B, and Phase C.

Result:
Passed


Integration Test 3:

Functions tested together:
-WriteReport() version 1.0, calculateRMS() version 1.0, calculateMean() version 1.0, peak_peak() version 1.0, and count_clipping() version 1.0

Test Steps:
1. Load the waveform data.
2. Separate Phase A, Phase B, and Phase C values.
3. Call the waveform calculation functions.
4. Write the calculated results into results.txt.

Expected Outcome:
-The program should calculate RMS voltage, DC offset, peak values, peak-to-peak voltage, and clipping events for each phase.
-The RMS voltage should be approximately 230 V.
-The DC offset should be approximately 0 V.
-Clipping events should be detected correctly.

Result:
Passed


Integration Test 4:

Functions tested together:
-main() version 1.0, LoadCSV() version 1.0, WriteReport() version 1.0, and all waveform analysis functions version 1.0

Test Steps:
1. Compile the complete program.
2. Run the executable.
3. Confirm that the CSV file is loaded.
4. Confirm that the report file is generated.
5. Check that the program exits successfully.

Expected Outcome:
-The full system should run from start to finish without crashing.
-The program should read the dataset, analyse the waveform, write results.txt, free memory, and display a completion message.

Result:
Passed


Integration Test Results Summary:

-The system correctly integrates main(), LoadCSV(), WriteReport(), and the waveform analysis functions.

-The CSV file was successfully loaded into the Dataset struct array, and the data was then processed by the report-writing function.

-The waveform functions correctly worked together with WriteReport() to produce RMS voltage, DC offset, peak voltage, peak-to-peak voltage, and clipping results.

-The full program successfully generated results.txt using the real dataset.

The following still needs to be resolved or implemented in future versions:
- Additional handling for corrupted CSV rows
- maybe an extension for variance and standard deviation
- and also an extension for bitwise phase status flags

- Evidence of Scenario Testing

Software/project version 1.0

Testing Scenario:

This scenario test evaluates the complete functionality of the Power Quality Waveform Analyser from loading the CSV file to generating the final report.

The purpose of this test was to confirm that the entire system works correctly when handling real waveform data and different user/file conditions.

Steps:

1. Input the CSV filename:

power_quality_log.csv

2. Validate input (successful).

The program successfully opened the CSV file.

3. Receive next input.

The program skipped the CSV header row and began reading waveform data.

4. Store waveform data into the Dataset struct array.

5. Process waveform calculations.

The program calculated:

- RMS voltage

- DC offset

- Peak voltage

- Peak-to-peak voltage

- Clipping events

- Average frequency

- Average power factor

- Average THD

6. Generate results.txt output file.

7. Display completion message to the user.

Expected Result:

The program should successfully process the dataset and generate a complete waveform analysis report.

Actual Result:

Passed.

4.1 Multiple Variations Tested

4.1.1 Valid CSV file – Passed

Scenario:

The correct CSV file containing 1000 waveform samples was provided.

Expected Result:

The program should successfully analyse the data and generate results.txt.

Actual Result:

Passed.

4.1.2 Missing CSV file – Passed

Scenario:

An incorrect filename was provided.

Expected Result:

The program should display an error message and terminate safely.

Actual Result:

Passed.

4.1.3 CSV file with header row – Passed

Scenario:

The CSV file included column headings in the first row.

Expected Result:

The program should skip the header row correctly and continue reading numerical data.

Actual Result:

Passed.

4.1.4 Waveform clipping detection – Passed

Scenario:

The dataset contained waveform values at the clipping threshold.

Expected Result:

The program should detect clipping events where |voltage| >= 324.9 V.

Actual Result:

Passed.

4.2 Empty CSV file – Failed Initially

Scenario:

An empty CSV file was tested.

Expected Result:

The program should display a warning that no valid samples were found.

Initial Result:

Failed.

Cause:

The program initially attempted calculations even when no samples existed.

Solution:

A validation check was added:

if (n > 0)

This prevented division by zero and invalid calculations.

Final Result After Fix:

Passed.

Scenario Testing Summary:

The program successfully completed all major functional scenarios using the real waveform dataset.

The system correctly handled:

- Valid waveform data

- Missing files

- CSV headers

- Clipping detection

- Full report generation

One issue involving empty files was discovered during testing and corrected by adding validation checks before performing calculations.

Future versions could include:

- Detection of corrupted CSV rows

- Automatic handling of multiple CSV files

- More advanced error reporting

- Evidence of Version Control

The project was managed using Git and GitHub for version control throughout development.

Version control was enabled using the built-in Git tools inside CLion.

Development Process:

1. Version Control Setup

Inside CLion, Version Control was enabled by selecting:

VCS → Enable Version Control Integration

Git was selected as the version control system.

Initially, Git was not installed on the computer, so CLion prompted for Git installation. After installing Git successfully, version control integration was enabled.

2. Creating Commits

The Git menu then became available inside CLion.

Project files were selected and committed using:

Git → Commit

During the first commit, Git requested:

- Git username

- Git email address

After entering the credentials, the commit completed successfully.

3. Pushing to GitHub

The project was uploaded to GitHub using:

Git → Push

CLion then requested:

- The remote repository link (origin)

- GitHub authentication

After authenticating through the web browser, the push completed successfully and all project files appeared in the GitHub repository.

4. Repository Management

The repository was updated throughout development with additional commits after debugging and improvements.

Examples of updates included:

- RMS calculation implementation

- CSV file loading

- DC offset bug fix

- Report formatting improvements

GitHub Repository Link:

https://github.com/davenlem111-source/Power-Quality-Waveform-Analyser.git

- Full Code Submission:
Zip File:

-main.c

-IO.c

-IO.h

-Waveform.c

-Waveform.h

-unit_tests.c

-integration_tests.c

-CMakeLists.txt

-README.md

-power_quality_log.csv

-test_logs.txt

-Development Logbook DOCX

- Design Rationale

The program was designed using modular programming principles.

The project was split into:
- main.c
- IO.c
- waveform.c

This improved readability, maintainability, and debugging.

The Dataset struct was designed to represent one complete row of waveform data, allowing related values such as voltage, frequency, current, and THD to remain grouped together.

Advanced statistical analysis was not included because the project focused on the waveform analysis requirements specified in the coursework brief.

- Pointer and Memory Management:

-Dynamic memory allocation was implemented using malloc():

-Dataset *data = malloc(MAX_SAMPLES * sizeof(Dataset));

-This allocates memory for 1000 Dataset records at runtime.

Pointers were used for:

- Passing arrays into functions

- Returning multiple values in peak_peak()

- Accessing dynamically allocated memory

-Memory was released using free(data) to prevent memory leaks.

- Testing Strategy:

The program was tested using:

- Valid CSV files

- Missing files

- Empty files

- Balanced waveform values

- Clipping threshold values

The RMS calculation was verified against the expected UK mains RMS value of approximately 230 V.

The clipping detection was validated using the ±324.9 V threshold.

Unit testing, integration testing, and scenario testing were also performed to verify overall system functionality.

- Debugging Challenges:

Bug 1 — CSV File Not Opening

Problem:

-The CSV file could not be located during execution.

Cause:

-CLion was using the wrong working directory.

Solution:

-The CSV file was moved into the correct directory and the CLion working directory was corrected.

Bug 2 — Incorrect DC Offset Values

Problem:

-The DC offset calculation produced extremely large values.

Cause:

-The mean calculation incorrectly squared the waveform values.

Incorrect code:

sum += values[i] * values[i];

Correct code:

sum += values[i];

After correction, the DC offset correctly approached 0 V.

Bug 3 — Missing Numerical Values in results.txt

Problem:

-Units appeared without numerical values.

Cause:

-fprintf() format specifiers were missing.

Solution:

-Formatting placeholders such as %.2f were added.

- References:

1. C Programming Language

Kernighan, B.W. and Ritchie, D.M. (1988) The C Programming Language. 2nd edn. Prentice Hall.

2. Git Documentation

Chacon, S. and Straub, B. (2014) Pro Git. Apress.

Git Documentation. Available at:
https://git-scm.com/doc

3. Software Testing

Kaner, C., Falk, J. and Nguyen, H. (1999) Testing Computer Software. 2nd edn. Wiley.

4. Coursework Brief

University of the West of England (2025) UGMFGT-15-1 Programming for Engineers Coursework Brief.